1. WebSocket Signaling Endpoint

Secure URL (wss://...) for establishing real-time signaling between our apps and Cloud Connect.

wss://sip2.cloud-connect.in:7443/

2. Authentication Method

The supported authentication mechanism (API key, JWT token, OAuth, etc.) and the process for client apps to securely connect.

Authentication is based on SIP REGISTER method that CloudConnect should receive over web-socket which will respond with 401 Unauthorized

Post that browser should replies with SIP Digest credentials.

3. Signaling Protocol & Message Format-

The JSON schema/protocol for events such as:

Registration expiry time (in seconds) and keep-alive/re-register intervals - 3600sec

Call invitation (offer) : Through INVITE method only

Call acceptance/rejection (answer) : Post Digest authentication as explained above.

ICE candidates exchange : Required in case of NAT only.

Call termination/end : It should manage at Webrtc endpoint only through BYE method.

Call status updates (ringing, busy, missed, etc.) : CloudConnect will provide the status at real time through SIP dialog.

4. Media Handling

STUN/TURN server details for NAT traversal.

If supported, information about media servers (SFU/MCU) for group calls or recording.

Supported codecs (audio/video) and any bandwidth recommendations.

PCMA and PCMU Codecs will support.

5. Call Control APIs

REST APIs or WebSocket commands for call management (initiate, hang up, mute/hold, logs, etc.).

It should manage at Webrtc endpoint only as per the standard SIP RFC.

6. Error Handling & Limits

Error codes / reconnection guidelines.

Limits on concurrent connections, call participants, and message/payload sizes.

All error codes will respond as per the standard SIP modules and concurrency will allow as per the subscription.

7. Additional Information

Any SDKs, sample code, or documentation you provide for Web/Flutter/.NET Core integration.

Unfortunately we can’t provide any reference code . You can explore the standard Open source like PJSIP , SIP.js etc.

Here is a concise technical overview and integration summary based on the provided CloudConnect WebRTC/SIP signaling documentation, suitable for developer consumption and planning:

Secure WebSocket Signaling Endpoint

Endpoint: wss://sip2.cloud-connect.in:7443/

Used for real-time SIP signaling between client apps and CloudConnect, adhering to secure (wss) transport.

Authentication Method

Employs SIP REGISTER with Digest Authentication over the WebSocket:

Client sends a REGISTER request.

Server responds with SIP 401 Unauthorized.

Client must retry REGISTER, including valid SIP Digest credentials.

No JWT, API key, or OAuth mechanism is involved.

Signaling Protocol & Message Format

Standard SIP Protocol (over WebSocket; follows RFC 3261):

Registration Expiry: 3600 seconds.

Keep-Alive/Re-Register: Every 3600 seconds.

Call Invitation: SIP INVITE method only.

Call Answer/Rejection: Sent post successful Digest authentication.

ICE Candidates: Required only for NAT traversal scenarios.

Call Termination: BYE method issued by the WebRTC endpoint.

Call Status (ringing/busy/missed): Real-time updates via standard SIP dialog states.

Message Format: Standard SIP messages (not JSON).

Media Handling

NAT Traversal: Integration with external STUN/TURN servers is required (details assumed to be configured by client or determined per scenario).

Media Server (SFU/MCU): Not explicitly provided; architect group/recording support independently if needed.

Supported Codecs: PCMA and PCMU (audio).

Bandwidth Recommendations: Not specified, use defaults for narrowband codecs unless otherwise advised.

Call Control APIs

No REST/WebSocket API specific to call control outside of standard SIP over WebSocket.

All call control (initiate, hang up, mute, hold) must be managed via SIP signaling handling in WebRTC endpoint, strictly following SIP RFC methods (INVITE, BYE, etc.).

Error Handling & Limits

Errors: Returned as standard SIP response codes (e.g., 401 Unauthorized, 486 Busy Here, 403 Forbidden, etc.).

Reconnection: Follow standard SIP/transport reconnection and retry logic.

Concurrency Limits: Governed by your CloudConnect subscription; check with CloudConnect for numerical thresholds on concurrent calls and message sizes.

Integration Resources

SDKs / Reference Code: None officially provided.

Recommended open-source libraries: PJSIP, SIP.js for SIP/WebRTC stack implementation.

No dedicated SDK, sample code, or vendor-maintained documentation for Web, Flutter, or .NET Core.

Key Points Table

| Aspect | Detail |
| --- | --- |
| WebSocket Endpoint | wss://sip2.cloud-connect.in:7443/ |
| Auth Mechanism | SIP REGISTER with Digest Authentication |
| Signaling Protocol | Standard SIP over WebSocket, RFC 3261 |
| Media NAT Traversal | Use external STUN/TURN servers, as needed |
| Codecs Supported | PCMA, PCMU (audio only) |
| SDKs/Code Provided | None; use SIP.js or PJSIP |
| Call Control | SIP signaling only |
| Errors/Concurrency | Standard SIP codes; subscription-based concurrency |

For practical implementation, leveraging an open SIP/WebRTC stack (like SIP.js for browsers or PJSIP for native platforms) will be essential, with all SIP logic handled per open standards for integration with CloudConnect.